### LG Key System

Software Release Note

LDK C.6Ak

02 Dec, 2005

| SYSTEM | IpLDK |
| --- | --- |
| SOFTWARE | MPB |
| VERSION | C.6Ak |
| CHECKSUM |  |
| ECO |  |

Version : ~ C.6Ah	3

Version : C.6Ai	7

## Version : ~ C.6Ah

| Feature | Description | IR No. | Applied Ver. | Report From | PP Ver. |  |
| --- | --- | --- | --- | --- | --- | --- |
| LDK-100 | Support LDK-100 (MPBN / MPBSN / ASMU1N / ASMU2N) -> These are developed by end of life of EDO DRAM on DMEMU. |  |  |  |  |  |
| IP Trunk | Support SIP and H.323 G/K. |  |  |  |  |  |
| CID SLT | Support CID SLT on CSLIB12 /CSLIU12 |  |  |  |  |  |
| VOIBE NAT | Support NAT on VOIBE. |  |  |  |  |  |
| IP Soft Phone | Call Log/SMS/Call Recording features on IP Soft Phone are supported. |  |  |  |  |  |
| Extension Name | If extension’s name is registered, name is displayed in idle LCD message. Ex ) STATION 100 -> Mr KIM 100 |  |  | India |  |  |
| Internal Call Parking | Support internal call parking. |  |  | India |  |  |
| UCD DND | A UCD member does not get a UCD call in designated time (PGM 191 – F21), this member go to UCD DND state automatically. This call is rerouted to other idle member, or queueing. When timer set to 000, feature is not activated. |  |  | Norway |  |  |
| Mobile Extension | Incorrect Operation of Mobile Extension Feature - If registered Mobile Phone Number is exceeded 10 digit, from 11’th digit , all digits are don’t care. For example, Mobile No 07748 980 006 and 07748 980 009 are recognized same number although 6 and 9 are different. -> It is fixed. | UK3464 |  | UK |  |  |
| Prepaid Call Fraction | SMDR fraction is applied to Prepaid call (MOROCO only) | MO040705 |  | MOROCO |  |  |
| DDI Name | DDI Name pass on Call Pick Up Calls - Call comes into group –ring/acd , call is ringing on station, other station dials pick up code and requires DDI name to appear on the screen. | IRE106 |  | Ireland |  |  |
| CO-to-CO call SMDR | When CO to CO Call is established, and Conversation is finished, LDK system SMDR have problem as below: + On-line case: LDK not problem + Off-line case: LDK have problem: not store transfer call data between sub 852002 - sub 852001. not store authorization code. only sent incoming call data -> CO-to-CO call is saved and printed. | VN0007 |  | Vietnam |  |  |
| Pre-selected Message | Pre selected message as voice prompt to external caller gives possibility to leave a message even with AAIB – Finland only. | FIN00434 |  | Finland |  |  |
| DID/DISA Destination | DID DISA Reroute Destination (PGM 167 – Btn 5/6/7) is applied to NZ. | NZ0465 |  | NZ |  |  |
| Walking COS | The `extended` bins of PGM 227 Authorization codes, ideally suited to Walking Class of Service usage. But Walking Class of Service still can only use bins belonging to the physical port count. -> Support Walking COS with extended bin. | NZ0466 |  | NZ |  |  |
| Station 000 | Support Station number ‘000’ |  |  | Korea |  |  |
| DISA | On-Demand mode in DISA is added. |  |  | Korea |  |  |
| VMIB Prompt | If 2 digit station number, 1 zero is added in front of sta number. f sta number is 21, it is not 021. This problem is also when sta is forwarded to sta that 2 digit sta number. “Forwarded to sta 021” -> It is fixed. | FIN00428 |  | Finland |  |  |
| Default Value | Unsupervised conference timer default value is changed to ‘4’. (Telkom Only) | 'MCSA-L300-0081 |  | Telkom |  |  |
| Attendant Call Queuing | If attendant presses CO btn while he is receiving intercom call, the intercom call is queued. (Telkom Only) | MCSA-L300-0080 |  | Telkom |  |  |
| Mobile Extension | Support Mobile-extension in Hunt Call.(Circular, Terminal, UCD group are supported) | NW0309A |  | Norway |  |  |
| Attendant Queuing | Audible indication is provided to attendant when intercom call is queued. | MCSA-L100-0099 |  | Telkom |  |  |
| Codec Type | G.729 Codec type can be programmed in PGM 382/Btn 5. If G.729 is assigned, only LIP-7000 terminal should be integrated to the system. (Caution: RSG/SoftPhone/LIP-24LD can be muted in G.729.) | IP LDK-0020 |  | Israel |  |  |
| COS Control | Day and Night COS of station can be changed with ‘$02 XXXX yy (XXXX=Station No. yy=COS’ command from terminal. |  |  |  |  |  |
| Call log of ACNR | After activating the ACNR, there is a problem of duplicate dialed number on Call Log (Dialed Call). Israel only. | IPLDK-0034 |  | Israel |  |  |
| Emergency Intrusion | Emergency Intrusion feature is added. Description: If Emergency Supervisor Station calls other station and the Supervisor is heard busy tone, the Supervisor can dial Emergency Intrusion code and makes conversation directly with the called station. Operation : Dial Emergency Intrusion code (ex “#1”) while listening busy tone. Related PGM: - Emergency Intrusion Code (PGM 109 – FLEX 7) - Emergency Supervisor (PGM 112 – FLEX 24) |  |  | Italy |  |  |
| LCR Dial Tone Detect | ‘LCR Dial Tone Detect (PGM161/Btn 22)’ is added. Description: If this value is set to ON, LDK system first checks if the CO provides dial tone in case if analog CO is seized for LCR dialing. If there’s no dial tone, the call is rerouted to Alternate DMT Index. If LCR type is set to M13, LCR dial tone detect option is not applied. |  |  | Italy |  |  |

## Version : C.6Ai

| Feature | Description | IR No. | Applied Ver. | Report From | PP Ver. |  |
| --- | --- | --- | --- | --- | --- | --- |
| NET CFW for ring assigned CO line | LDK supports Net CFW feature about the ring assigned CO line. | RU-051003-40 |  |  |  |  |
| Numbering Plan | Station Range is not deleted at PGM 105. (Emergency Intrusion Default value disjoints with some of number sets.) => fixed. (Emergency Intrusion default value is changed) | NZ-0481, AUS-0275 |  | NZ, AUS |  |  |
| UCD DND Tmr | UCD DND Timer (PGM 191 – F21) : Second based -> changed to Minute based. |  |  |  |  |  |
| Music Source | Station Group Music Source range : changed to include “Hold tone”. | NZ-0235 |  |  |  |  |
| CSLIB24 naming | Display “CSLIB24” is Changed to “CSLIB12E”. |  |  | IT, SAF, CIS |  |  |
| GK Find Port | GK Find Port is not changed with PCAdm and has abnormal value |  |  | SPAIN |  |  |
| CPT Tone Detect | In case of Analog CO Call, To Avoid CO line locking, CPT Busy or Error Tone is detected when outside user disconnected. | TR0003Ring |  | Turkey |  |  |
| Hunt Group Transfer | UCD 620 (100) Station 100, 101 = DKT 1. Place a DID call to 620 and answer the call on 100. 2. Place a DID call to 101 and transfer the call 100.  Problem: This CO will recall to 101 immediately. -> This problem is fixed | SW2512 |  | Sweden |  |  |

## Version : C.6Aj

| Feature | Description | IR No. | Applied Ver. | Report From | PP Ver. |  |
| --- | --- | --- | --- | --- | --- | --- |
| HUNT DND LED | Ring Group is set to 620 (100, 101) Sta 100, Flex 24 (HUNT DND) = Trans + 87 + 620 Sta 101, Flex 24 = 100 - Logout 100 by pressing flex 24 on sta 100. - Flex 24 on 101 starts to flash. - Go off-hook and then on-hook on sta 100. - Flex 24 on 101 don't flash any more. ->This problem is fixed | SW2520 | 3.6Aj | Sweden |  |  |
| Alt If No Member of Circular Group | If a Circular member is set to DND or Preselect Msg and Alt if no member option is set to ON. Then a call is placed to that circular member, call is not rerouted to overflow destination. -> This problem is fixed | SW2521 | 3.6Aj | Sweden |  |  |
| Tapi ring event | Send TAPI ring event for new UCD group calls not to all group member but only to a ringing station -> Australia only | AUS0232 | 3.6Aj | Australia |  |  |
| ISDN INFO MSG with LCR | Do not send information message to PX when a user dial codes which are not matched with LCR table -> UK, Italy only | UK3467 | 3.6Aj | UK |  |  |
| DID incoming call | If Station number conflicts with digits of its COLP table, the station cannot receive DID call. -> fixed. | SE-L300-001 | 3.6Aj | Korea |  |  |
| Transferred call ORI CLI | R2 CO call is transferred to another R2 CO after answered. Instead of ORI CLI, CFW CLI is sent. => fixed. | SE-L300-002 | 3.6Aj | Korea |  |  |

## Version : C.6Ak

| Feature | Description | IR No. | Applied Ver. | Report From | PP Ver. |  |
| --- | --- | --- | --- | --- | --- | --- |
| Tenancy Group Auto Ring Mode | Auto Ring mode of Tenancy Groups does not follow Weekly Time Table of same entries. (Tenancy Group numbers and Weekly Time Table index does not matched.) ->Fixed. | NZ0497 | C.6Ak | NZ |  |  |
| Default Value | Some of Australian Default values are changed. | AUS0271 | C.6Ak | AUS |  |  |
| Display text problem | 1 DIAL BY ICM NAME 2 DIAL BY STA SPD NAME 3 DIAL BY SYS SPD NAME This message should be changed the existing Danish text into: 1. KALD VIA NAVN – should be corrected to: KALD VIA LOKAL NAVN. 2. KALD VIA KORTNR. – should be corrected to: KALD VIA EGNE KORTNR 3. KALD VIA KORTNR. – should be corrected to: KALD VIA SYS KORTNR. -> fixed | DM0313 | 3.6Aj | Denmark |  |  |
| Individual UCD member’s unanswered calls show 65534 calls | When viewing the statistics of an individual UCD agent, sometimes the unanswered calls counter shows 65534 calls. At Supervisor station, press ACD STATUS button, sometimes, each agent unanswered call item is shown 65534, or 65535. | MCSA-L300/600-0075 | 3.6Aj | South Africa |  |  |